ECA18 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

ENGINEERING DESIGN REPORT

Course Outcome: CO1 – Apply the concepts of guided modes in step-index and graded-index fibers, evaluate losses, dispersion and compensation techniques, including OSNR.

Assessment Type: Open-Ended Engineering Design Assignment

COURSE CODE : ECA1803

COURSE NAME : OPTOELECTRONICS AND OPTICAL SYSTEM

NAME: THARIKA.S

REG NO: 192412031

THE ENGINEERING DESIGN PROBLEM

A communication service provider plans to establish a 60 km optical fiber link operating at

1550 nm to carry 10 Gb/s traffic between a central office and a business building, with

provision for a later upgrade to 100 Gb/s. The link must use standard ITU-T fiber categories,

tolerate tight bends at the building entry, and maintain pulse integrity, polarization

performance and signal quality. The design engineer must determine whether the proposed

link configuration is technically suitable before deployment.

The proposed design must satisfy the following information and constraints:

ABSTRACT:

This report presents the engineering design and evaluation of a 60 km optical fiber communication link operating at 1550 nm with a data rate of 10 Gb/s and provision for a future upgrade to 100 Gb/s. The design process begins with the communication requirements and proceeds through the investigation of fiber alternatives, guided-mode analysis, optical power budgeting, dispersion compensation, polarization-mode dispersion (PMD) evaluation, optical signal-to-noise ratio (OSNR) analysis, integrated link design and final deployment assessment.

Two candidate step-index fibers are evaluated using numerical aperture, normalized frequency and guided-mode characteristics. Standard single-mode fiber is selected for long-distance, high-speed transmission, while bend-insensitive fiber is considered for building entry. The total specified distributed attenuation is calculated as 0.18 dB/km. Connector, splice, bending and dispersion-compensation losses are included in the link budget.

Chromatic dispersion is evaluated for G.652.D and G.655 fiber categories, and the required dispersion-compensating fiber (DCF) length is calculated. PMD and birefringence are also examined to determine their implications for reliable transmission and future upgrading. The amplification strategy is evaluated using OSNR calculations to establish whether the required signal quality can be maintained in the presence of amplified spontaneous emission (ASE) noise.

The final configuration integrates the selected fiber, dispersion-management arrangement, optical amplifier and receiver. Analytical verification and a proposed OptiSystem simulation methodology are used to assess the design. The report concludes with a technically justified deployment decision, identifying the conditions that must be satisfied before the link can be approved for practical installation.

Keywords: Optical fiber communication, G.652.D, G.655, G.657.A, numerical aperture, V-number, optical power budget, chromatic dispersion, DCF, PMD, OSNR, OptiSystem.

1. INTRODUCTION

Optical fiber communication is widely used in telecommunications, enterprise networks and high-speed data transmission because it provides high bandwidth, low electromagnetic interference and reliable long-distance communication. Information is transmitted in the form of modulated optical signals through a dielectric waveguide consisting of a core and a cladding.

The performance of an optical communication link depends on several interacting parameters, including fiber attenuation, guided modes, chromatic dispersion, polarization-mode dispersion, connector and splice losses, receiver sensitivity and optical signal-to-noise ratio. A link may provide sufficient received power but still experience unacceptable signal degradation because of dispersion or noise. Therefore, optical link design requires a combined evaluation of power, temporal pulse integrity and signal quality.

In this assignment, a communication service provider requires a 60 km optical fiber link operating at 1550 nm to carry 10 Gb/s traffic between a central office and a business building. The system must tolerate tight bends at the building entrance and support a potential future upgrade to 100 Gb/s.

The engineering task is to investigate alternative fiber configurations, perform the required calculations, select appropriate components, integrate them into a complete link and verify whether the resulting configuration meets the specified performance requirements.

1.1 Aim

To design and critically evaluate a standards-compliant 60 km optical fiber communication link operating at 1550 nm for 10 Gb/s transmission, with adequate optical power, controlled dispersion, acceptable PMD and OSNR, bend tolerance and a technically credible path toward 100 Gb/s operation.

1.2 Objectives

- Interpret the communication requirements and identify the relevant optical transmission parameters.

- Calculate the numerical aperture, V-number, guided-mode characteristics and cutoff wavelength of the candidate fibers.

- Compare single-mode, step-index multimode and graded-index multimode propagation.

- Calculate the distributed fiber attenuation, total link loss, received optical power and system margin.

- Evaluate chromatic dispersion, pulse broadening, DCF length and additional compensation loss.

- Assess PMD, birefringence, beat length and their implications for high-speed transmission.

- Calculate the OSNR before and after amplification and evaluate the amplification strategy.

- Develop an integrated optical link and verify it using analytical calculations and, where available, OptiSystem simulation.

- Make a final deployment recommendation based on the combined results.

1.3 Problem Statement

A 60 km optical communication link must transmit 10 Gb/s data at 1550 nm while maintaining adequate received power, acceptable pulse integrity and sufficient OSNR. The link must also accommodate installation bends and provide a feasible upgrade path to 100 Gb/s. The available fiber alternatives and system parameters have different advantages and limitations. Consequently, a technically justified design must be developed by comparing alternatives, calculating the expected performance and verifying the compatibility of the selected components.

1.4 Scope of the Design

The scope includes:

- Optical fiber selection and propagation-mode analysis.

- Fiber attenuation and practical link power budgeting.

- Chromatic dispersion and dispersion-compensation design.

- PMD and polarization-related analysis.

- OSNR and optical amplifier evaluation.

- Integrated link configuration and verification.

- Deployment suitability and future-upgrade considerations.

The analysis uses the numerical values provided in the assignment. Where practical parameters are not specified, such as receiver overload limits, exact amplifier noise figure, transmitter linewidth or connector return loss, the limitations are explicitly identified rather than assumed to be verified.

2. ENGINEERING DESIGN PROCESS

The proposed link is developed using the following systematic engineering design process:

REQUIREMENT → ALTERNATIVES → ANALYSIS → SELECTION → LINK DESIGN → VERIFICATION → DEPLOYMENT DECISION

Step 1: Requirement

To Identify the required distance, wavelength, data rate, receiver sensitivity, system margin, bend tolerance and future upgrade capability.

Step 2: Alternatives

To identify feasible alternatives for fiber type, propagation mode, dispersion management, amplification and polarization management.

Step 3: Analysis

To Use appropriate optical communication equations to evaluate each alternative quantitatively. Compare the calculated values with the stated design limits.

Step 4: Selection

To Choose the most suitable configuration based on technical performance, compatibility, installation requirements and future scalability.

Step 5: Link Design

Combine the selected transmitter, fiber, dispersion-compensation section, amplifier and receiver into a complete optical communication system.

Step 6: Verification

To Compare the design targets with analytical results and simulation measurements. Identify all requirements that pass, fail or remain unverified.

Step 7: Deployment Decision

To Determine whether the proposed link should be deployed, conditionally accepted for further testing or redesigned.

This approach ensures that the final configuration is created from the communication requirements rather than selected without engineering justification.

STAGE 1 – INTERPRET THE COMMUNICATION REQUIREMENT

3.1 Requirement Analysis

The first stage establishes the functional and technical requirements of the optical link before selecting any components.

| Parameter | Given requirement |
| --- | --- |
| Transmission distance | 60 km |
| Operating wavelength | 1550 nm |
| Initial data rate | 10 Gb/s |
| Future data rate | 100 Gb/s |
| Transmitter power | +3 dBm |
| Receiver sensitivity | −20 dBm |
| Required system margin | 6 dB |
| Source spectral width | 0.4 nm |
| Input pulse width | 30 ps |
| Number of connectors | 4 |
| Connector loss | 0.4 dB each |
| Number of splices | 10 |
| Splice loss | 0.08 dB each |
| Required OSNR | 17 dB in 0.1 nm reference bandwidth |
| Building-entry bend loss | 0.8 dB for G.652.D; 0.1 dB for G.657.A |

The design must provide sufficient received power while maintaining acceptable pulse integrity and signal quality. The bend-sensitive installation environment makes G.657.A a strong candidate for the building-entry section. The long transmission distance and high data rate favor single-mode propagation.

3.2 Purpose of the 1550 nm Operating Wavelength

The wavelength of 1550 nm corresponds to the low-attenuation region of standard silica optical fiber. It is widely used in long-distance communication systems because it provides favorable transmission loss and compatibility with optical amplifiers such as erbium-doped fiber amplifiers.

The relationship between wavelength and optical frequency is:

f=cλ

where c=3×108m/s and λ=1550×10-9m.

f=3×1081550×10-9
f≈193.55 THz

The operating frequency lies within the conventional telecommunications C-band region.

Although the wavelength is suitable for long-distance communication, attenuation and dispersion must still be evaluated using the selected fiber parameters.

3.3 Ray Theory and Wave Theory

Light propagation through an optical fiber can be studied using ray optics and wave optics.

Ray theory treats light as rays undergoing refraction and total internal reflection at the core-cladding boundary. It is useful for understanding light acceptance, numerical aperture and the general propagation of multimode light when the core is large relative to the wavelength.

Wave theory treats light as an electromagnetic field governed by Maxwell’s equations. It is necessary to determine guided modes, field distributions, cutoff conditions and polarization behavior accurately.

For single-mode fiber, wave-optical analysis is particularly important because only the fundamental spatial mode is guided. For multimode fiber, ray theory can provide an intuitive approximation, but modal dispersion and detailed mode behavior require wave-based analysis.

3.4 Light Acceptance and Guided Modes

The numerical aperture (NA) describes the light-acceptance capability of an optical fiber. A higher NA generally permits light to enter the fiber over a wider acceptance cone.

The normalized frequency, or V-number, combines the wavelength, core radius and refractive-index contrast to determine the number and nature of supported guided modes.

For a step-index fiber:

V=2πaλNA

For a weakly guiding step-index fiber, the approximate single-mode condition is:

V<2.405

Below this cutoff, the fiber supports only the fundamental spatial mode. Above it, additional guided modes can propagate.

3.5 Fiber Losses to Be Controlled

The main specified distributed attenuation mechanisms are:

- Absorption loss: Optical energy is absorbed by the fiber material and impurities.

- Rayleigh scattering loss: Microscopic refractive-index fluctuations scatter optical energy.

- Bending loss: Curvature can cause optical power to radiate from the guided mode.

Connector and splice losses are discrete losses rather than distributed fiber attenuation. They must be added separately to the link budget.

3.6 Dispersion and Pulse Integrity

Dispersion causes different spectral or modal components of an optical pulse to arrive at different times.

- Modal dispersion occurs when multiple guided modes have different propagation delays.

- Chromatic dispersion occurs because different wavelength components travel at different group velocities.

- Polarization-mode dispersion occurs because orthogonal polarization components can have different group delays.

Single-mode propagation removes intermodal dispersion, but chromatic dispersion and PMD still need to be evaluated.

3.7 OSNR and Signal Quality

OSNR is the ratio of optical signal power to optical noise power within a defined reference bandwidth. It is used to assess the signal quality in the presence of ASE noise generated by optical amplifiers.

A low OSNR can reduce the receiver’s decision margin, worsen the eye opening and increase the bit error rate (BER). A satisfactory received-power level alone is therefore insufficient to establish acceptable communication performance.

3.8 Practical Constraints

The final configuration must account for:

- The required 60 km transmission distance.

- Tight building-entry bends.

- The specified receiver sensitivity and 6 dB system margin.

- Dispersion compensation and the additional loss it introduces.

- Amplifier noise and OSNR.

- Polarization behavior and PMD.

- Future operation at 100 Gb/s.

3.9 Design Requirement Statement

The proposed optical link shall transmit 10 Gb/s traffic over 60 km at 1550 nm using an appropriate single-mode ITU-T fiber configuration. It shall maintain a received power of at least −14 dBm to satisfy the −20 dBm receiver sensitivity with the required 6 dB system margin, maintain OSNR of at least 17 dB in a 0.1 nm reference bandwidth, control pulse broadening through appropriate dispersion management, tolerate the specified building-entry bends and undergo separate verification before any 100 Gb/s upgrade is approved.

STAGE 2 – INVESTIGATE AND SELECT THE FIBER / PROPAGATION CONFIGURATION

4.1 Candidate Fiber Parameters

The assignment specifies two candidate step-index fibers.

| Parameter | Fiber A | Fiber B |
| --- | --- | --- |
| Core refractive index, n1 | 1.4500 | 1.480 |
| Cladding refractive index, n2 | 1.4450 | 1.465 |
| Core radius, a | 4 μm | 25 μm |
| Wavelength, λ | 1.55 μm | 1.55 μm |

The numerical aperture and V-number are calculated to determine the light-acceptance capability and modal behavior of each fiber.

4.2 Numerical Aperture Calculation

For a step-index fiber in air:

NA=n12-n22

Fiber A

Given:

n1=1.4500, n2=1.4450
NAA=1.45002-1.44502
NAA=2.1025-2.088025
NAA≈0.1203

Fiber B

Given:

n1=1.480, n2=1.465
NAB=1.4802-1.4652
NAB=2.1904-2.146225
NAB≈0.2102

Interpretation

Fiber B has a higher numerical aperture and therefore a wider acceptance cone than Fiber A. However, a higher NA does not automatically mean better performance for long-distance digital communication. The number of guided modes and the resulting modal dispersion must also be considered.

4.3 V-Number Calculation

The normalized frequency is:

V=2πaλNA

Fiber A

VA=2π41.550.1203
VA≈1.95

Since:

1.95<2.405

Fiber A supports only the fundamental spatial mode at 1550 nm under the weak-guidance approximation.

Fiber B

VB=2π251.550.2102
VB≈21.30

Since:

21.30>2.405

Fiber B supports multiple guided modes and is therefore multimode at the operating wavelength.

4.4 Approximate Number of Guided Modes in Fiber B

For a step-index multimode fiber with a sufficiently large V-number, the approximate number of guided modes is:

Mstep≈V22

For graded-index fiber with an approximately parabolic refractive-index profile:

Mgraded≈V24

Using VB=21.30:

VB2≈453.69

For step-index propagation:

Mstep≈453.692
Mstep≈227

For an ideal parabolic graded-index profile with the same V-number:

Mgraded≈453.694
Mgraded≈113

These are approximate mode counts under the stated idealized models. The actual number of guided modes depends on the refractive-index profile and modal counting convention.

4.5 Cutoff Wavelength of Fiber A

The single-mode cutoff condition for a step-index fiber is:

Vc=2.405

Using:

V=2πaλNA

the cutoff wavelength is:

λc=2πaNA2.405

Substituting Fiber A parameters:

λc=2π40.12032.405
λc≈1.257 μm

The operating wavelength is 1.55 μm, which is greater than the calculated cutoff wavelength. Therefore, Fiber A operates in the single-mode regime at 1550 nm.

4.6 Comparison of Propagation Alternatives

| Characteristic | Single-mode step-index | Step-index multimode | Graded-index multimode |
| --- | --- | --- | --- |
| Core size | Small | Large | Large |
| Guided modes | Fundamental spatial mode | Many modes | Many modes |
| Modal dispersion | Absent ideally | High | Reduced |
| Long-distance bandwidth | High | Limited by modal dispersion | Better than step-index multimode |
| Alignment tolerance | More demanding | Less demanding | Less demanding |
| Suitability for 60 km at 10 Gb/s | High | Poor for this application | Less suitable than single-mode |
| 100 Gb/s upgrade potential | Strong, with appropriate transceivers and dispersion management | Limited | Limited compared with single-mode |

Single-mode fiber is preferred because it eliminates intermodal dispersion and supports high-bandwidth, long-distance communication. Graded-index multimode fiber reduces modal dispersion by using a refractive-index profile that compensates for differences in modal path length and propagation velocity. Nevertheless, it remains less suitable than single-mode fiber for the specified distance and data rate.

4.7 Comparison of ITU-T Fiber Categories

G.652.D: Standard single-mode fiber suitable for general telecommunications and long-distance transmission. Its dispersion at 1550 nm is given as 17 ps/(nm·km).

G.655: Non-zero dispersion-shifted single-mode fiber with a lower specified dispersion magnitude of 4.5 ps/(nm·km). It can reduce chromatic-dispersion broadening, but the supplied data do not specify its complete attenuation and bend-loss characteristics.

G.657.A: Bend-insensitive single-mode fiber suitable for access networks and building-entry installations. Its specified entry-bend loss in this assignment is 0.1 dB, compared with 0.8 dB for G.652.D.

G.657.A and G.652.D are not mutually exclusive in the sense of being completely different modal solutions: both are single-mode fiber categories. A practical link can use standard single-mode fiber for the main route and bend-insensitive fiber for the building-entry section, provided the selected fiber variants and their optical compatibility are confirmed.

4.8 Analysis of Alternatives

Alternative 1 is Fiber A operated as single-mode fiber. It provides the appropriate modal behavior and is compatible with the long-distance application.

Alternative 2 is Fiber B operated as step-index multimode fiber. Its large V-number supports approximately 227 modes under the stated approximation, increasing the potential for modal dispersion.

Alternative 3 is graded-index multimode fiber. Its refractive-index profile reduces modal dispersion compared with step-index multimode fiber, but the application still favors single-mode propagation.

Alternative 4 is a single-mode link using G.652.D or G.655 for the main route, with G.657.A used where tight building-entry bends are present. This alternative combines long-distance single-mode performance with improved bend tolerance.

4.9 Design Decision 1 – Selected Fiber Configuration

Selected configuration: Single-mode propagation using a G.652.D-compatible main transmission route and G.657.A bend-insensitive fiber at the building entry. G.655 remains a dispersion-oriented alternative for further evaluation.

The selection is supported by the following results:

- Fiber A has V≈1.95<2.405, confirming single-mode operation at 1550 nm.

- Fiber B has V≈21.30, indicating multimode propagation.

- Single-mode propagation avoids intermodal dispersion.

- G.657.A has the lower specified building-entry bend loss.

- The single-mode configuration provides a more appropriate foundation for 10 Gb/s transmission over 60 km and for a future high-speed upgrade.

Design decision statement: I recommend a single-mode fiber configuration using G.652.D-compatible transmission fiber and G.657.A for the bend-sensitive building entry because it provides suitable long-distance modal performance while reducing the specified entry-bend loss.

STAGE 3 – DEVELOP AND EVALUATE THE OPTICAL POWER BUDGET

5.1 Purpose of the Power Budget

The optical power budget determines whether enough optical power reaches the receiver after accounting for distributed fiber attenuation, connectors, splices, bends and other passive components.

The received optical power must be greater than or equal to the receiver sensitivity plus the required system margin.

Given:

PT=+3 dBm
Psensitivity=-20 dBm
Mrequired=6 dB

Therefore, the minimum acceptable received power is:

PR,min=Psensitivity+Mrequired
PR,min=-20+6
PR,min=-14 dBm

The available loss budget without amplification is:

PT-PR,min=3--14
Lmax=17 dB

5.2 Total Distributed Fiber Attenuation

The specified distributed losses are:

| Loss mechanism | Attenuation |
| --- | --- |
| Absorption | 0.04 dB/km |
| Rayleigh scattering | 0.11 dB/km |
| Distributed bending | 0.03 dB/km |
| Total | 0.18 dB/km |

The total attenuation coefficient is:

αtotal=αabs+αRayleigh+αbend
αtotal=0.04+0.11+0.03
αtotal=0.18 dB/km

For a 60 km link:

Lfiber=αtotalL
Lfiber=0.1860
Lfiber=10.8 dB

5.3 Connector Loss

Four connectors are specified, each with a loss of 0.4 dB.

Lconnectors=NcLc
Lconnectors=40.4
Lconnectors=1.6 dB

5.4 Splice Loss

Ten splices are specified, each with a loss of 0.08 dB.

Lsplices=NsLs
Lsplices=100.08
Lsplices=0.8 dB

5.5 Link Budget Using G.652.D-Type Building Entry

The specified building-entry bend loss is 0.8 dB.

The total link loss is:

Ltotal=Lfiber+Lconnectors+Lsplices+Lentry bend
Ltotal=10.8+1.6+0.8+0.8
Ltotal,G.652.D=14.0 dB

Received power is:

PR=PT-Ltotal
PR=3-14.0
PR=-11.0 dBm

The receiver sensitivity margin is:

M=PR-Psensitivity
M=-11--20
M=9.0 dB

The excess margin above the required 6 dB is:

Mexcess=9-6
Mexcess=3.0 dB

Therefore, the baseline G.652.D-type link meets the specified 6 dB margin before adding DCF or any other unlisted losses.

5.6 Link Budget Using G.657.A Building Entry

The specified building-entry bend loss is 0.1 dB.

Ltotal=10.8+1.6+0.8+0.1
Ltotal,G.657.A=13.3 dB

Received power is:

PR=3-13.3
PR=-10.3 dBm

The receiver sensitivity margin is:

M=-10.3--20
M=9.7 dB

The excess margin above the required 6 dB is:

Mexcess=9.7-6
Mexcess=3.7 dB

Comparison of Baseline Power Budgets

| Parameter | G.652.D entry | G.657.A entry |
| --- | --- | --- |
| Distributed fiber loss | 10.8 dB | 10.8 dB |
| Connector loss | 1.6 dB | 1.6 dB |
| Splice loss | 0.8 dB | 0.8 dB |
| Entry-bend loss | 0.8 dB | 0.1 dB |
| Total link loss | 14.0 dB | 13.3 dB |
| Received power | −11.0 dBm | −10.3 dBm |
| Margin above sensitivity | 9.0 dB | 9.7 dB |
| Required margin | 6 dB | 6 dB |
| Baseline result | Pass | Pass |

5.7 Dominant Loss Mechanism

The Rayleigh scattering attenuation is 0.11 dB/km, which is greater than the specified absorption and distributed bending attenuation.

Over 60 km:

LRayleigh=0.1160
LRayleigh=6.6 dB

Absorption loss is:

Labs=0.0460=2.4 dB

Distributed bending loss is:

Lbend=0.0360=1.8 dB

Thus, Rayleigh scattering is the largest specified distributed fiber-loss mechanism.

5.8 Practical Loss-Reduction Measures

Measure 1: Use bend-insensitive fiber at the building entry.
G.657.A reduces the specified entry-bend loss from 0.8 dB to 0.1 dB, improving the link budget by 0.7 dB.

Measure 2: Optimize connectors and splices.
Use clean connectors, proper inspection and cleaning procedures, high-quality fusion splicing, and suitable installation practices to reduce avoidable insertion losses and reflections.

Additional practical measures include maintaining the manufacturer-specified bend radius, avoiding mechanical stress and measuring installed attenuation using optical test equipment.

5.9 Effect of Dispersion Compensation on the Power Budget

The DCF attenuation is 0.5 dB/km. As calculated in Stage 4, the G.652.D compensation length is 10.2 km.

LDCF=0.510.2
LDCF=5.1 dB

Adding this loss to the baseline link:

For G.652.D-type entry:

Ltotal,DCF=14.0+5.1=19.1 dB
PR=3-19.1
PR=-16.1 dBm

Margin:

M=-16.1--20=3.9 dB

For G.657.A-type entry:

Ltotal,DCF=13.3+5.1=18.4 dB
PR=3-18.4
PR=-15.4 dBm

Margin:

M=-15.4--20=4.6 dB

Both configurations fall short of the required 6 dB margin if the DCF is added without amplification or another loss-budget improvement.

5.10 Design Decision 2 – Selected Power-Budget Configuration

The selected baseline arrangement uses G.657.A at the bend-sensitive building entry and compatible single-mode fiber over the main route. However, the complete DCF-equipped configuration must include a power-restoration strategy.

The calculated baseline received power is −10.3 dBm for the G.657.A entry arrangement, providing a 9.7 dB sensitivity margin. The addition of the 10.2 km DCF reduces the predicted received power to −15.4 dBm, leaving only 4.6 dB of sensitivity margin.

Therefore, the dispersion-compensated link does not meet the required 6 dB system margin without further intervention. An amplifier after the DCF is a candidate solution, but its gain, ASE noise, OSNR, receiver overload limit and operational stability must be verified.

Design decision statement: I recommend a single-mode power-budget configuration using G.657.A at the building entry, with DCF loss explicitly included and a verified amplification strategy selected to restore the required receiver margin without violating the OSNR requirement.

STAGE 4 – INVESTIGATE AND SELECT THE DISPERSION / POLARIZATION-MANAGEMENT CONFIGURATION

6.1 Requirement Analysis

The selected optical link must transmit 10 Gb/s data over 60 km while maintaining adequate pulse integrity. At 10 Gb/s, each bit occupies a time interval of 100 ps. If dispersion causes excessive pulse broadening, adjacent pulses can overlap and produce inter-symbol interference (ISI).

The design must therefore compare the dispersion performance of G.652.D and G.655, calculate the required compensation length, evaluate the additional DCF attenuation and determine whether PMD and polarization management are acceptable.

6.2 Modal Dispersion

Modal dispersion occurs when different guided modes travel through a multimode fiber with different propagation delays.

In an ideal single-mode fiber, only the fundamental spatial mode propagates, so intermodal dispersion is absent. However, this does not eliminate chromatic dispersion or PMD.

In graded-index multimode fiber, the refractive index decreases gradually from the core center toward the cladding. Rays traveling longer paths through lower-index regions can travel faster than rays following shorter paths through higher-index regions. This partially equalizes modal arrival times and reduces modal dispersion.

For the selected single-mode configuration, modal dispersion is not the primary limitation. Chromatic dispersion and PMD must still be considered.

6.3 Chromatic Dispersion in G.652.D Fiber

Chromatic-dispersion pulse broadening is approximated by:

Δτ=∣D∣ΔλL

where:

- Dis the chromatic-dispersion coefficient.

- Δλis the source spectral width.

- Lis the fiber length.

For G.652.D:

D=17 ps/(nm\cdotpkm)
Δλ=0.4 nm
L=60 km

Substituting:

Δτ652=170.460
Δτ652=408 ps

This broadening is substantially greater than the 100 ps bit period.

6.4 Chromatic Dispersion in G.655 Fiber

For G.655:

D=4.5 ps/(nm\cdotpkm)
Δτ655=4.50.460
Δτ655=108 ps

G.655 provides significantly lower calculated chromatic-dispersion broadening than G.652.D under the given parameters.

6.5 Combining Input Pulse Width and Dispersion Broadening

The input pulse width is 30 ps. Assuming the input pulse width and dispersion-induced broadening can be treated as independent contributions, the approximate output pulse width is calculated using the root-sum-square (RSS) relationship:

τout=τin2+Δτ2

G.652.D

τout,652=302+4082
τout,652=900+166464
τout,652≈409.1 ps

Comparison with the bit period:

409.1 ps>100 ps

The estimated pulse width exceeds four bit periods, indicating severe pulse overlap under this simplified model.

G.655

τout,655=302+1082
τout,655=900+11664
τout,655≈112.1 ps

Comparison with the bit period:

112.1 ps>100 ps

The G.655 result is substantially better, but the estimated pulse width still exceeds the bit period. Consequently, G.655 alone cannot be declared sufficient using this pulse-width comparison.

These RSS values are approximate engineering estimates. A complete system evaluation must also consider the source waveform, modulation format, receiver filtering and the convention used to define pulse width.

6.6 Condition for First-Order Dispersion Compensation

The total accumulated chromatic dispersion is:

Dtotal=DfLf+DDCFLDCF

For ideal first-order compensation:

DfLf+DDCFLDCF=0

Rearranging:

LDCF=-DfLfDDCF

For G.652.D:

Df=17 ps/(nm\cdotpkm)
Lf=60 km
DDCF=-100 ps/(nm\cdotpkm)

Substituting:

LDCF=-1760-100
LDCF=10.2 km

The positive length indicates the amount of negative-dispersion fiber required to cancel the accumulated positive dispersion in the ideal first-order model.

6.7 Verification of Residual Dispersion

The accumulated dispersion in the transmission fiber is:

DfLf=1760=1020 ps/nm

The accumulated dispersion in the DCF is:

DDCFLDCF=-10010.2
DDCFLDCF=-1020 ps/nm

Therefore:

Dresidual=1020-1020
Dresidual=0 ps/nm

This represents ideal first-order compensation. Practical residual dispersion can differ because of component tolerances, wavelength dependence, temperature and fiber variations.

The DCF compensates chromatic dispersion but does not eliminate attenuation, PMD or every higher-order dispersion effect.

6.8 Additional DCF Loss and Received Power

The DCF attenuation is 0.5 dB/km.

LDCF loss=αDCFLDCF
LDCF loss=0.510.2
LDCF loss=5.1 dB

Using the G.657.A building-entry configuration:

Ltotal=13.3+5.1
Ltotal=18.4 dB

Received power without amplification is:

PR=3-18.4
PR=-15.4 dBm

The sensitivity margin is:

M=-15.4--20
M=4.6 dB

The required margin is 6 dB, so the DCF-equipped passive link fails the power-margin requirement by:

6-4.6=1.4 dB

This result is important because it demonstrates that dispersion compensation cannot be selected independently of the power budget.

6.9 Alternative Dispersion-Management Configurations

Alternative 1: Uncompensated G.652.D

This configuration has a baseline received power of −11 dBm for the G.652.D entry case, but the estimated pulse broadening is 408 ps. It is unsuitable under the given pulse-width requirement without further dispersion management.

Alternative 2: G.655 with reduced chromatic dispersion

G.655 reduces the calculated broadening to 108 ps. However, this is still comparable to or greater than the 100 ps bit period. Its full link budget and residual dispersion must be evaluated before deployment.

Alternative 3: G.652.D with DCF

A 10.2 km DCF section theoretically cancels the first-order accumulated chromatic dispersion. Its additional 5.1 dB loss reduces the passive link margin below the specified requirement.

Alternative 4: Dispersion compensation with power restoration

This configuration combines the 60 km single-mode transmission fiber, the calculated DCF length and an amplifier placed after the DCF. It offers a candidate solution for restoring received power, subject to OSNR and receiver operating-range verification.

6.10 PMD Calculation

The specified PMD coefficient is:

DPMD=0.1 ps/km

The statistical link PMD estimate is:

τPMD=DPMDL

For L=60km:

τPMD=0.160
τPMD≈0.775 ps

PMD Limit at 10 Gb/s

The bit period is:

Tb=11010=100 ps

The specified PMD limit is:

τlimit=0.1Tb
τlimit=0.1100
τlimit=10 ps

Since:

0.775 ps<10 ps

the calculated PMD satisfies the stated 10 Gb/s limit.

PMD Limit at 100 Gb/s

For a 100 Gb/s data rate:

Tb=11011=10 ps
τlimit=0.110
τlimit=1 ps

Since:

0.775 ps<1 ps

the calculated PMD also satisfies the assignment’s nominal 100 Gb/s PMD limit.

However, the result leaves a much smaller margin for the upgrade. Actual PMD performance is statistical and may vary with fiber installation, environmental conditions and the deployed fiber characteristics. A 100 Gb/s deployment therefore requires a separate standards- and transceiver-specific verification.

6.11 Birefringence and Beat Length

Birefringence occurs when the effective refractive indices experienced by two orthogonal polarization states differ.

The beat length is:

LB=λB

where Bis the birefringence parameter given in the assignment.

At 1550 nm:

λ=1.55×10-6 m

Standard Fiber

B=1×10-7
LB=1.55×10-61×10-7
LB=15.5 m

Polarization-Maintaining Fiber

B=4×10-4
LB=1.55×10-64×10-4
LB=3.875×10-3 m=3.875 mm

Polarization-maintaining (PM) fiber has deliberately introduced birefringence that helps preserve a selected polarization state when the optical input is aligned correctly. Its beat length is correspondingly much shorter.

PM fiber is generally used in systems that require controlled polarization, such as certain interferometric sensors and specialized coherent optical assemblies. It is not automatically required for an ordinary single-mode communication link. For the proposed link, appropriate low-PMD transmission fiber and compatible transceivers are the primary requirements.

6.12 PMD-Mitigation Technique

A practical PMD-management strategy is to specify low-PMD transmission fiber and use compatible optical transceivers. During installation, unnecessary mechanical stress and sharp bends should be avoided. For systems where residual PMD becomes significant, adaptive electronic compensation or polarization-domain compensation may be considered, depending on the transceiver architecture.

6.13 Design Decision 3 – Selected Dispersion and PMD Configuration

The preferred design uses single-mode transmission fiber with explicit dispersion compensation. For the G.652.D dispersion coefficient supplied in the assignment, the calculated DCF length is 10.2 km.

The calculated DCF loss is 5.1 dB, which causes the passive power margin to fall to 4.6 dB with G.657.A at the building entry. Therefore, the DCF-equipped design must incorporate a verified power-restoration arrangement.

G.655 provides a useful alternative because its calculated uncompensated pulse broadening is lower. However, its estimated 112.1 ps output pulse width still exceeds the bit period in the supplied simplified model. It should not be selected as a guaranteed compensation-free solution without further system analysis.

The PMD estimate of 0.775 ps meets the stated 10 Gb/s and 100 Gb/s limits, although the 100 Gb/s margin is limited.

Design decision statement: I recommend a single-mode dispersion-management configuration using a calculated 10.2 km DCF section for the G.652.D dispersion case, with the additional attenuation restored through a verified amplifier arrangement and PMD controlled through suitable low-PMD fiber and compatible transceivers.

STAGE 5 – EVALUATE OSNR AND THE AMPLIFICATION STRATEGY

7.1 Requirement Analysis

An optical amplifier increases signal power, but it also introduces ASE noise. Consequently, an amplifier cannot be evaluated solely by its gain. Its effect on OSNR must be calculated to determine whether the required signal quality is maintained.

The assignment specifies:

- Signal power: −16 dBm.

- ASE noise power: −39 dBm.

- Required OSNR: 17 dB in a 0.1 nm reference bandwidth.

- Signal gain: 14 dB.

- ASE-noise increase: 16 dB.

7.2 Definition of OSNR

Optical signal-to-noise ratio is defined as:

OSNR=PsignalPnoise

In decibel form:

OSNRdB=Psignal,dBm-Pnoise,dBm

The signal and noise powers must be measured using the same specified reference bandwidth. In this assignment, the reference bandwidth is 0.1 nm.

7.3 Initial OSNR Calculation

Given:

Psignal=-16 dBm
PASE=-39 dBm

Therefore:

OSNRinitial=-16--39
OSNRinitial=23 dB

Comparison with the required OSNR:

23 dB>17 dB

The initial OSNR exceeds the requirement by:

23-17=6 dB

7.4 Signal Power After Amplification

The amplifier provides 14 dB of signal gain.

Psignal,out=Psignal,in+G
Psignal,out=-16+14
Psignal,out=-2 dBm

7.5 ASE Noise Power After Amplification

The specified ASE-noise increase is 16 dB.

PASE,out=PASE,in+16
PASE,out=-39+16
PASE,out=-23 dBm

These calculations use the assignment’s stated noise increase. They do not independently derive the amplifier’s ASE noise from a physical noise-figure model.

7.6 OSNR After Amplification

OSNRout=Psignal,out-PASE,out
OSNRout=-2--23
OSNRout=21 dB

The change in OSNR is:

ΔOSNR=21-23
ΔOSNR=-2 dB

Although the signal power increases by 14 dB, the noise power increases by 16 dB. Consequently, the OSNR decreases by 2 dB.

7.7 OSNR Verification

| Parameter | Initial | After amplifier | Requirement |
| --- | --- | --- | --- |
| Signal power | −16 dBm | −2 dBm | System-dependent |
| ASE noise power | −39 dBm | −23 dBm | System-dependent |
| OSNR | 23 dB | 21 dB | At least 17 dB |
| OSNR margin | 6 dB | 4 dB | Non-negative |

The calculated post-amplifier OSNR is 21 dB, which exceeds the specified minimum of 17 dB by 4 dB.

Therefore, the amplification strategy passes the assignment’s OSNR comparison using the provided signal-gain and noise-increase values.

7.8 Relationship Between OSNR, Q-Factor and BER

OSNR is an important optical signal-quality indicator, but it does not directly establish the exact BER without additional system information.

In a digital optical receiver, signal and noise affect the statistical separation between the logical one and zero levels. A higher Q-factor generally indicates better separation between these levels and typically corresponds to a lower BER under the applicable receiver model.

Inadequate OSNR can cause:

- Reduced eye opening.

- Increased uncertainty in receiver decisions.

- Lower Q-factor.

- Higher BER.

- Reduced effective system margin.

- Greater sensitivity to additional transmission impairments.

The exact OSNR required to achieve a particular BER depends on the modulation format, receiver architecture, filtering, coding and other system parameters.

7.9 Practical OSNR-Improvement Measures

Measure 1: Select an amplifier with an appropriate noise figure.
The amplifier should provide sufficient gain while introducing the least practical ASE noise. Its noise figure and gain characteristics should be included in the system model.

Measure 2: Optimize amplifier placement and avoid unnecessary amplification stages.
An amplifier placed after the DCF can restore power lost in the compensation section. The resulting signal and ASE levels must be checked to confirm that the OSNR remains acceptable.

Other measures include controlling reflections, avoiding excessive cascaded amplification and verifying OSNR using an optical spectrum analyzer.

7.10 Design Decision 4 – Selected OSNR and Amplification Strategy

The calculated initial OSNR is 23 dB, and the post-amplifier OSNR is 21 dB. The latter exceeds the specified minimum of 17 dB by 4 dB.

The 14 dB amplifier provides a useful candidate for restoring power after the DCF, but its actual suitability depends on the combined link simulation and component specifications. In particular, the supplied OSNR calculation must be reconciled with the actual signal and ASE powers at the amplifier input and output.

The amplifier’s output power must also remain within the receiver’s acceptable operating range. If the simulated signal power is too high, attenuation or a different gain setting may be required.

Design decision statement: I recommend retaining the 14 dB amplification setting as the initial candidate for evaluation because the supplied parameters predict a post-amplifier OSNR of 21 dB, which satisfies the 17 dB requirement. Final acceptance is conditional on verifying the actual power budget, ASE noise, receiver operating range and BER in the complete link model.

STAGE 6 – VERIFY AND CRITICALLY EVALUATE THE COMPLETE LINK

8.1 Objective of Complete-Link Verification

The purpose of this stage is to determine whether the individual design decisions made in Stages 2–5 can operate together as a complete optical communication system.

A link may satisfy its individual fiber, dispersion or OSNR calculations but still fail when all losses and impairments are considered simultaneously. Therefore, the final design must be evaluated using a common system configuration.

The verification process includes:

- Confirming that the selected fiber supports the intended propagation mode.

- Recalculating total link loss after adding dispersion compensation.

- Verifying the received optical power and required system margin.

- Checking residual chromatic dispersion and pulse integrity.

- Evaluating PMD against the specified limits.

- Confirming that OSNR exceeds the required minimum.

- Identifying parameters that require simulation or practical measurement before deployment.

8.2 Final Integrated Link Configuration

The proposed configuration consists of a single-mode optical transmitter operating at 1550 nm, a 60 km transmission fiber, a DCF section, an optical amplifier and a receiver with performance-monitoring instruments.

The main transmission route is designed around G.652.D-compatible single-mode fiber, with G.657.A used at the building-entry section where tight bends are expected.

The DCF length is calculated from the G.652.D dispersion value provided in the assignment. The amplifier is provisionally placed after the DCF to restore the optical power lost during dispersion compensation.

Figure 1. Proposed Optical Fiber Link Block Diagram

Note: The diagram is a conceptual design. The physical order and implementation of the G.657.A entry section, DCF and amplifier must be matched to the actual route and selected components. A practical link may use a short G.657.A patch or entry section rather than replacing the entire 60 km route.

8.3 Final Adopted Design Parameters

| Design parameter | Proposed value |
| --- | --- |
| Transmission distance | 60 km |
| Operating wavelength | 1550 nm |
| Initial data rate | 10 Gb/s |
| Future upgrade target | 100 Gb/s, subject to separate verification |
| Main fiber | G.652.D-compatible single-mode fiber |
| Building-entry fiber | G.657.A bend-insensitive fiber |
| Launch power | +3 dBm |
| Distributed attenuation used in the supplied budget | 0.18 dB/km |
| Connector loss | 1.6 dB total |
| Splice loss | 0.8 dB total |
| Building-entry bend loss | 0.1 dB for G.657.A |
| Baseline link loss | 13.3 dB |
| Baseline received power | −10.3 dBm |
| DCF length for the G.652.D dispersion case | 10.2 km |
| DCF attenuation | 0.5 dB/km |
| DCF insertion loss from the specified attenuation | 5.1 dB |
| Passive received power with DCF | −15.4 dBm |
| Initial amplifier setting | 14 dB gain |
| Calculated post-amplifier OSNR from supplied OSNR data | 21 dB |
| Required OSNR | At least 17 dB in 0.1 nm |
| Required receiver margin | 6 dB |

The table represents the proposed starting configuration. The actual deployed design must account for any additional insertion losses, component tolerances and receiver operating limits.

8.4 Final Optical Power Verification

The baseline link loss using G.657.A at the building entry is:

Lbaseline=13.3 dB

The DCF adds 5.1 dB:

Lpassive,total=13.3+5.1
Lpassive,total=18.4 dB

The received power before amplification is:

PR=3-18.4
PR=-15.4 dBm

The margin is:

M=-15.4--20
M=4.6 dB

The minimum acceptable power is −14 dBm. Thus, the passive DCF-equipped link fails the required system-margin criterion by 1.4 dB.

If the specified 14 dB amplifier gain is applied after the DCF, the idealized output power estimate becomes:

Pout=-15.4+14
Pout=-1.4 dBm

The corresponding margin relative to receiver sensitivity is:

M=-1.4--20
M=18.6 dB

This calculation shows that the stated amplifier gain can restore power in the simplified model. However, −1.4 dBm is only a predicted power level, not a guaranteed acceptable receiver input. The receiver overload limit and the actual amplifier output, including noise and component losses, must be verified.

Furthermore, the OSNR data supplied in Stage 5 form a separate signal/noise calculation. They should not automatically be treated as the measured OSNR of this exact integrated link unless the simulated signal and noise powers reproduce the same operating conditions.

8.5 Final Verification Table

| Requirement | Specified target | Calculated result | Status |
| --- | --- | --- | --- |
| Modal condition | Single-mode transmission | Fiber A: V≈1.95 | Pass |
| Building-entry bend tolerance | G.657.A entry loss of 0.1 dB | 0.1 dB specified | Pass on supplied assumption |
| Baseline fiber and component loss | Sufficient power margin | 13.3 dB total | Pass before DCF |
| Power margin with DCF | At least 6 dB | 4.6 dB without amplifier | Fail |
| Power margin with 14 dB gain | At least 6 dB | 18.6 dB in the idealized power calculation | Conditional; receiver overload must be checked |
| G.652.D chromatic dispersion | Pulse integrity at 10 Gb/s | 408 ps uncompensated broadening | Fail without compensation |
| G.652.D DCF compensation | First-order dispersion cancellation | 10.2 km; ideal residual 0 ps/nm | Pass theoretically |
| G.655 pulse broadening | Compatible with 100 ps bit period | Approximately 112.1 ps under the RSS estimate | Fail under the simplified comparison |
| PMD at 10 Gb/s | At most 10 ps | Approximately 0.775 ps | Pass nominally |
| PMD at 100 Gb/s | At most 1 ps | Approximately 0.775 ps | Pass nominally; upgrade verification required |
| Initial OSNR | At least 17 dB | 23 dB | Pass |
| OSNR after amplifier | At least 17 dB | 21 dB using supplied noise values | Pass for the supplied calculation |
| End-to-end BER | Must meet transceiver/service target | Not yet measured | Unverified |
| Receiver input operating range | Must remain within specified limits | Receiver overload limit not provided | Unverified |
| 100 Gb/s operation | Must be demonstrated | Not established by the 10 Gb/s analysis | Unverified |

8.6 Critical Evaluation of the Complete Link

The verification results show that the selected single-mode fiber configuration is appropriate for the distance and modal requirements. The baseline optical power budget also satisfies the required margin.

However, the dispersion analysis identifies a major limitation. The uncompensated G.652.D broadening is 408 ps, which is much greater than the 100 ps bit period. The calculated 10.2 km DCF section cancels the accumulated first-order chromatic dispersion, but introduces 5.1 dB of additional attenuation.

The DCF-equipped passive link consequently fails the required power margin. The proposed amplifier can restore the power in the simplified calculation, but it also introduces ASE noise and requires receiver-range verification.

The OSNR calculation gives 21 dB after amplification, which exceeds the required 17 dB. Nevertheless, this result does not establish the BER of the complete link. An end-to-end simulation must include the selected fiber, DCF, amplifier, transmitter, receiver and noise settings together.

The PMD calculation satisfies the nominal limits at both 10 Gb/s and 100 Gb/s. However, the 100 Gb/s limit is only 1 ps, so the calculated 0.775 ps PMD value leaves a comparatively small margin. The future upgrade requires a separate assessment of the actual transceiver format, receiver sensitivity, dispersion tolerance and PMD performance.

8.7 Final Verification Conclusion

The proposed configuration is analytically promising but cannot yet be considered fully verified for deployment. The design satisfies the single-mode criterion and the supplied OSNR calculation, and its PMD estimate meets the stated nominal limits. However, the DCF introduces a power-margin deficit in the passive link, and the final BER, receiver operating range and actual integrated-link OSNR remain to be verified.

Therefore, the complete link should proceed to integrated simulation and component-level verification before a deployment decision is finalized.

FINAL SYSTEM DESIGN – K6 CREATE / DESIGN

9.1 Integrated Design Objective

The objective of the final design is to combine the selected fiber, power budget, dispersion compensation, amplification and polarization-management arrangement into a coherent 60 km optical communication link.

The design is based on the results obtained from the earlier stages rather than treating each parameter independently.

9.2 Selected System Architecture

The selected architecture contains the following functional sections:

- Optical transmitter: Generates the data-bearing optical signal at 1550 nm with a launch power of +3 dBm.

- Main transmission fiber: Uses compatible single-mode fiber over the 60 km route.

- Bend-insensitive entry section: Uses G.657.A fiber at the building entrance to reduce the specified entry-bend loss.

- Dispersion compensation: Uses a calculated 10.2 km DCF section for the G.652.D dispersion case.

- Optical amplifier: Uses 14 dB as the initial gain setting for evaluating power restoration and OSNR.

- Optical receiver: Converts the received optical signal to an electrical signal and recovers the transmitted data.

- Performance monitoring: Measures received power, OSNR, eye opening and BER.

9.3 Design Integration and Compatibility

The selected components must be evaluated as a complete system because each component influences the operating conditions of the next stage.

The transmission fiber contributes attenuation and chromatic dispersion. The DCF reduces the accumulated dispersion but adds attenuation. The amplifier compensates for power loss but introduces ASE noise. The receiver must operate within its specified input-power range and achieve the required BER under the resulting OSNR and dispersion conditions.

Therefore, the design is not accepted merely because each component has an appropriate individual specification. Its suitability depends on the combined link performance.

9.4 Integrated Link Equations

The total passive loss is:

Lpassive=αfLf+Lconnectors+Lsplices+Lbends+αDCFLDCF

For the selected configuration:

Lpassive=0.1860+1.6+0.8+0.1+0.510.2
Lpassive=18.4 dB

The estimated received power before amplification is:

PR=PT-Lpassive
PR=-15.4 dBm

If an amplifier provides a net 14 dB gain:

Pout=PT-Lpassive+G
Pout=3-18.4+14
Pout=-1.4 dBm

This expression is a first-order power estimate. Actual component insertion losses, amplifier saturation and receiver input limits must be included in the final model.

9.5 Dispersion Integration

For the G.652.D transmission fiber:

Dfiber,total=1760=1020 ps/nm

For the DCF:

DDCF,total=-10010.2=-1020 ps/nm

Thus:

Dresidual=0 ps/nm

The selected DCF length theoretically compensates the accumulated first-order dispersion. The actual simulated residual dispersion must be checked at the operating wavelength and across the relevant signal bandwidth.

9.6 OSNR Integration

The supplied OSNR data produce:

OSNRinitial=23 dB
OSNRafter amplifier=21 dB

The calculated post-amplifier OSNR is above the specified 17 dB requirement.

For the integrated design, the OSNR must be measured or simulated using the actual optical signal and noise powers at the receiver. The separate assignment values should be used as a verification reference, not as a substitute for the final end-to-end result.

9.7 Provision for a 100 Gb/s Upgrade

The initial link is designed for 10 Gb/s, but the upgrade to 100 Gb/s requires additional engineering validation.

At 100 Gb/s, the bit period is approximately 10 ps. The dispersion and PMD tolerances depend on the selected modulation format, transceiver technology, spectral width and receiver architecture. A configuration that performs adequately at 10 Gb/s may not satisfy the requirements at 100 Gb/s.

The upgrade should therefore include:

- Selection of 100 Gb/s-compatible transceivers.

- Verification of the actual fiber dispersion and PMD specifications.

- Evaluation of receiver sensitivity and required OSNR.

- Assessment of the modulation format and spectral width.

- End-to-end BER and eye-diagram testing.

- Verification that the existing amplifier and dispersion-management components remain compatible.

The proposed single-mode architecture provides a suitable foundation for the upgrade, but actual 100 Gb/s performance must be demonstrated.

9.8 Final Integrated Design Decision

The integrated design uses a 60 km single-mode transmission route, G.657.A at the bend-sensitive entry, a 10.2 km DCF section for first-order G.652.D dispersion compensation and a 14 dB amplifier as the initial power-restoration candidate.

This arrangement addresses modal suitability, bending, dispersion and the initial power-margin deficit. Its OSNR calculation satisfies the supplied requirement, but complete-link BER, receiver operating range and integrated OSNR must be verified before deployment.

The design is therefore considered a proposed engineering configuration pending end-to-end verification, rather than a fully validated commercial link.

PRACTICAL IMPLEMENTATION USING OPTISYSTEM

10.1 Purpose of Simulation

OptiSystem can be used to model the proposed optical communication link and evaluate the combined effects of attenuation, dispersion, amplification and receiver noise. It is an appropriate tool for testing the design before practical deployment.

The simulation should be built incrementally, beginning with the baseline 10 Gb/s link and then adding the DCF and amplifier.

10.2 Proposed OptiSystem Components and Parameters

| Component | Proposed setting | Purpose |
| --- | --- | --- |
| PRBS Generator | Bit rate: 10 Gb/s | Generates the digital data sequence |
| NRZ Pulse Generator | Bit period: 100 ps | Converts the data into electrical pulses |
| CW Laser | Wavelength: 1550 nm; power: +3 dBm | Optical carrier generation |
| Mach–Zehnder Modulator | Suitable for 10 Gb/s NRZ operation | Modulates the optical carrier |
| Optical Fiber | Length: 60 km; attenuation: 0.18 dB/km | Models the transmission fiber |
| DCF / Dispersion Compensation Fiber | Length: 10.2 km; dispersion: −100 ps/(nm·km); attenuation: 0.5 dB/km | Compensates the G.652.D first-order dispersion |
| Optical Amplifier | Gain: 14 dB as the initial setting | Restores optical power |
| PIN Photodetector | Suitable for the receiver data rate | Converts optical power to electrical current |
| Low-Pass Filter | Suitable bandwidth for the selected receiver | Limits out-of-band electrical noise |
| BER Analyzer | Configured for the selected bit rate | Evaluates BER and eye performance |
| Optical Spectrum Analyzer | OSNR reference bandwidth: 0.1 nm | Evaluates optical signal and noise |
| Optical Power Meter | At selected link checkpoints | Measures optical power |

The exact component names and parameter fields depend on the installed OptiSystem version. The table specifies the required functional behavior and starting values rather than assuming every version uses identical labels.

10.3 Simulation Procedure

Step 1: Build the baseline transmitter.
Connect the PRBS Generator, NRZ Pulse Generator, CW Laser and Mach–Zehnder Modulator. Configure the data rate to 10 Gb/s and the wavelength to 1550 nm.

Step 2: Add the transmission fiber.
Configure the fiber length to 60 km and the attenuation to 0.18 dB/km. Use the appropriate dispersion parameter for the selected fiber case.

Step 3: Measure the baseline link.
Place an optical power meter after the transmission fiber. Compare the simulated received power with the analytical baseline budget. Account for any additional losses modeled by the software.

Step 4: Add the DCF.
Insert a DCF section with a length of 10.2 km, dispersion coefficient of −100 ps/(nm·km) and attenuation of 0.5 dB/km. Check whether the residual accumulated first-order dispersion is close to zero.

Step 5: Add the amplifier.
Configure the initial gain to 14 dB. Set the amplifier noise parameters using available component specifications. Measure the signal power and OSNR after the amplifier.

Step 6: Configure the receiver.
Connect the photodetector, electrical filter and BER Analyzer. Ensure that the receiver settings are suitable for the chosen 10 Gb/s modulation and detection scheme.

Step 7: Evaluate performance.
Record the eye diagram, BER, received optical power and OSNR. Repeat the analysis for the uncompensated link and the compensated link to demonstrate the effects of the DCF and amplifier.

Step 8: Perform sensitivity checks.
Vary amplifier gain, DCF settings and relevant receiver parameters within realistic ranges. Determine whether the link remains compliant when component values differ from their nominal settings.

10.4 Simulation Acceptance Criteria

| Measurement | Acceptance criterion |
| --- | --- |
| Operating wavelength | 1550 nm |
| Data rate | 10 Gb/s |
| Received optical power | At least −14 dBm for the specified 6 dB sensitivity margin |
| OSNR | At least 17 dB in a 0.1 nm reference bandwidth |
| Residual first-order dispersion | Near zero for the ideal compensated G.652.D case |
| PMD | Within the specified link limit |
| BER | Meets the selected transceiver or service requirement |
| Eye diagram | Adequate eye opening for reliable detection |
| Receiver input power | Within the manufacturer’s specified operating range |

The BER criterion cannot be assigned an exact numerical target from the supplied assignment because a specific BER threshold is not provided. The target should be chosen according to the selected transceiver or service specification and documented in the final report.

10.5 Optisystem implementation image

10.6 BER Results

10.6 Practical Limitations of OptiSystem

OptiSystem can demonstrate the modeled optical performance, but a successful simulation does not establish every installation requirement.

Actual deployment also requires confirmation of:

- Manufacturer-specified fiber attenuation and dispersion.

- Connector and splice insertion losses.

- Minimum bend radius and installed bend loss.

- Amplifier noise figure, saturation and output power.

- Receiver sensitivity and overload limits.

- Actual PMD and polarization behavior.

- Compatibility with the future 100 Gb/s transceiver.

The final report should clearly distinguish analytical calculations from simulated results. If the simulation has not been executed, the results must be described as predicted values or acceptance targets rather than measured simulation outcomes.

FINAL ENGINEERING DECISION – WOULD YOU DEPLOY THE DESIGN?

11.1 Deployment Decision

Decision: NO – NOT YET APPROVED FOR DEPLOYMENT.

The proposed configuration is technically promising, but the available calculations do not establish complete deployment readiness. The design should proceed to integrated OptiSystem verification and component-level validation before installation is approved.

11.2 Technical Justification

1. Fiber and modal suitability

Fiber A has a calculated V-number of approximately 1.95, satisfying the single-mode condition at 1550 nm. Fiber B has a V-number of approximately 21.30 and supports multiple guided modes. Therefore, single-mode transmission is the appropriate choice for the 60 km, 10 Gb/s application.

2. Optical power budget

The baseline G.657.A-entry link has a total loss of 13.3 dB and a received power of −10.3 dBm, giving a 9.7 dB margin above receiver sensitivity.

However, adding the calculated 10.2 km DCF introduces 5.1 dB of loss. The resulting passive received power is −15.4 dBm, providing only 4.6 dB of margin. This is below the required 6 dB margin, so the passive compensated link is not acceptable without further power-budget improvement.

3. Dispersion and compensation

The G.652.D uncompensated broadening is 408 ps, significantly greater than the 100 ps bit period. The 10.2 km DCF section theoretically cancels the accumulated first-order dispersion but increases attenuation. The compensated configuration must therefore be evaluated with its amplifier and receiver as one system.

4. PMD and polarization

The calculated PMD is approximately 0.775 ps. It meets the assignment’s stated 10 Gb/s and 100 Gb/s limits, but the 100 Gb/s limit leaves less margin. Actual fiber PMD and transceiver compatibility must be confirmed before upgrading.

5. OSNR and amplification

The calculated OSNR decreases from 23 dB to 21 dB after amplification. The result remains above the required 17 dB threshold, but the exact integrated-link OSNR must be measured or simulated with the selected amplifier and actual signal/noise levels.

6. Future 100 Gb/s upgrade

The single-mode architecture provides a suitable basis for future high-speed transmission. However, the 10 Gb/s design calculations alone do not prove 100 Gb/s readiness. The selected transceivers, modulation format, dispersion tolerance, receiver sensitivity and BER must be evaluated separately.

11.3 Required Redesign and Verification Actions

Before deployment, the following actions are required:

- Retain the single-mode architecture and use G.657.A at the bend-sensitive building entry.

- Include the 10.2 km DCF loss in the complete power budget.

- Verify the amplifier’s gain, ASE noise, output power and OSNR using the selected component model.

- Confirm that the receiver input remains within its specified operating range.

- Run the complete link in OptiSystem and record the eye diagram, BER, received power and OSNR.

- Verify the installed fiber’s attenuation, bend tolerance, PMD and connector/splice losses.

- Conduct a separate 100 Gb/s feasibility analysis before committing to the upgrade.

These actions address the identified shortcomings without discarding the useful results of the existing design.

11.4 Conditions for Deployment Approval

The design can be reconsidered for deployment when the complete link demonstrates all of the following:

- Received power and system margin meet the specified limits.

- OSNR remains at or above 17 dB in the required reference bandwidth.

- Dispersion and PMD are within the selected transceiver’s tolerances.

- BER meets the documented service requirement.

- The receiver operates within its permitted optical input range.

- Fiber, connector, splice and building-entry installation requirements are satisfied.

- The 100 Gb/s upgrade path is supported by a separate engineering verification.

Final decision statement: The proposed single-mode optical link is suitable as a design candidate for 60 km, 10 Gb/s transmission. However, deployment approval is withheld until the DCF-related power deficit is resolved, the integrated amplifier/OSNR behavior is verified, and the end-to-end receiver performance is demonstrated. The 100 Gb/s upgrade must be validated separately.

12. CONCLUSION

This report developed an optical fiber link design for 60 km transmission at 1550 nm with an initial data rate of 10 Gb/s and a future 100 Gb/s upgrade objective. The design followed the engineering sequence of requirement definition, alternatives, analysis, selection, link integration, verification and deployment decision.

The numerical aperture and V-number calculations established that Fiber A supports single-mode propagation at the operating wavelength, whereas Fiber B is multimode. Single-mode transmission was selected because it eliminates intermodal dispersion and provides a suitable basis for long-distance communication.

The specified distributed attenuation was calculated as 0.18 dB/km, producing 10.8 dB of fiber loss over 60 km. The baseline link with G.657.A at the building entry has a total loss of 13.3 dB and a received power of −10.3 dBm, satisfying the required 6 dB margin before dispersion compensation.

The dispersion analysis showed that G.652.D produces 408 ps of estimated uncompensated broadening, while G.655 produces 108 ps. A 10.2 km DCF section theoretically compensates the accumulated first-order dispersion in the G.652.D case. However, the DCF introduces 5.1 dB of additional attenuation, reducing the passive link margin to 4.6 dB. Therefore, power restoration must be included and verified as part of the complete design.

The calculated PMD was approximately 0.775 ps, within the nominal limits specified for both data rates. The OSNR analysis produced 23 dB before amplification and 21 dB after amplification, satisfying the specified minimum of 17 dB under the supplied noise assumptions.

The final design combines the single-mode transmission route, bend-insensitive building-entry fiber, dispersion compensation and a candidate 14 dB amplifier. Nevertheless, the calculated results do not independently prove end-to-end BER compliance, receiver compatibility or readiness for 100 Gb/s.

Consequently, the design is recommended for further simulation and engineering verification, but not yet approved for deployment. This conclusion reflects the central principle of optical-link engineering: the final decision must be based on the performance of the integrated system rather than on isolated component calculations.

REFERENCES

- International Telecommunication Union, ITU-T Recommendation G.652: Characteristics of a Single-Mode Optical Fibre and Cable.

- International Telecommunication Union, ITU-T Recommendation G.655: Characteristics of a Non-Zero Dispersion-Shifted Single-Mode Optical Fibre and Cable.

- International Telecommunication Union, ITU-T Recommendation G.657: Characteristics of a Bending-Loss Insensitive Single-Mode Optical Fibre and Cable.

- Gerd Keiser, Optical Fiber Communications, McGraw-Hill.

- Govind P. Agrawal, Fiber-Optic Communication Systems, Wiley.

- Optiwave Systems Inc., OptiSystem Optical Communication System Design Software Documentation.