Experiment 15 : DESIGN AND PERFORMANCE ANALYSIS OF PASSIVE OPTICAL NETWORK (PON) USING OPTISYSTEM

OBJECTIVES :

  - To design a wavelength-division multiplexed passive optical network (WDMPON) using OptiSystem.

  - To transmit multiple optical channels through a common single-mode fiber and passive optical splitter.

  - To evaluate the received signal quality using eye diagrams, Q-factor and biterror rate (BER).

SOFTWARE USED : Optisystem

CIRCUIT DIAGRAM:

RESULTS:

REFLECTION:

I learned to design and simulate a WDM-based passive optical network (PON) using Optisystem and transmit multiple wavelength channels through a shared optical fiber and passive splitter.

I evaluated the received signal quality using eye diagrams, Q-factor and BER understanding how optical losses affect network performance

RUBRICS:

| Objective | Assessment Criterion | Excellent – 4 | Good – 3 | Satisfactory – 2 | Needs Improvement – 1 | Weighta ge |
| --- | --- | --- | --- | --- | --- | --- |
| System Design & WDM-PON Setup | Correct selection and connection of OptiSystem components for the WDM-PON (PRBS Generators, NRZ Generators, CW Lasers, 2×1 WDM MUX, SMF, 1×2 Passive Splitter, PIN Photodetectors with filters, BER Analyzers). | All transmitters, WDM multiplexer, fiber, splitter, receivers, and analyzers are correctly selected, connected in sequence, and configured without errors. | All components are present with minor connection or layout errors, but the network remains functional. | Component selection is partially complete; minor connection or configuratio n errors impede signal flow on one or more channels. | Incorrect component selection or improper setup leading to complete simulation failure. | 20% |
| Parameter Configurati on & Execution | Accurate setup of input parameters: bit rate (2.5 Gb/s), NRZ- OOK, laser wavelengths (1550.12 nm, 1550.92 nm), power (10 dBm), SMF length (20 km), attenuation (0.2 dB/km), dispersion (16.75 ps/nm/km), and splitter loss (6.02 dB). | All block parameters are accurately set and the simulation is executed for all channels. | Parameters are mostly accurate with minor setting discrepancie s. | Basic parameters are configured, but multiple channel or fiber settings are missing. | Incorrect parameter settings or failure to run the simulation for the required channels. | 20% |
| Measureme nt & Eye Diagram Observation | Accurate extraction and recording of Max. Q-Factor, Min. BER, Eye Height, Threshold, and Decision Instant from each BER Analyzer. | Precise data extraction for all BER Analyzers, neatly tabulated, with clear eye diagrams captured for each channel. | Data is collected with minor omissions in Q-factor, BER, or eye parameters. | Partial data collection with incomplete tables or missing eye diagrams. | Inaccurate, incomplete, or missing data and measurements. | 20% |
| Channel Performanc e & Link Impairment Analysis | Detailed evaluation of per-channel performance and the effects of fiber attenuation, chromatic dispersion, and splitter loss on Qfactor, BER, and eye opening. | Comprehensive analysis correctly relating splitter loss, attenuation, and dispersion to Q- factor reduction, BER increase, and channel-to-channel variation. | Clear analysis identifying basic trends in Q-factor and BER across channels. | Basic attempt at result analysis without explaining mechanisms such as splitter loss or dispersion. | Erroneous or missing analysis of simulation results and eye diagrams. | 20% |
| Report & Conclusion | Identification of best and worst performing channels, assessment of whether BER targets are met, and structural clarity. | Report features clear eye diagrams, complete output tables, and insightful conclusions comparing channel quality and network limits. | Satisfactory report with relevant eye diagrams and sound concluding summary. | Incomplete report with missing diagrams, tables, or weak conclusions. | Disorganized or incomplete report lacking required diagrams and valid conclusions. | 20% |

CONCLUSION:

A WDM-PON system was successfully designed and tested using OptiSystem. Multiple wavelengths were multiplexed, transmijed through fiber, split, and detected at the receivers. Q-factor, BER, and eye-diagram results confirmed channel performance and showed the effects of ajenuation, dispersion, and splijer loss, verifying the operation of the WDM-PON network.