EXP 09: ANALYSIS OF PHOTODETECTOR NOISE AND SNR USING MATLAB/SCILAB

OBJECTIVES :

1. Analyze the key noise sources in an optical photodetector, namely shot noise, dark current noise and thermal (receiver) noise.

2. Compute the individual and total noise current variances (σ²) across a range of input optical powers.

3. Evaluate the Signal-to-Noise Ratio (SNR) and observe its scaling in the thermalnoise-limited and shot-noise-limited regimes.

4. Calculate the Minimum Detectable Optical Power (Pmin) at SNR = 1 (0 dB). 5. Simulate and visualize photodetector noise and SNR curves using MATLAB.

SOFTWARE USED : MATLAB

PROGRAM:

% Photodetector Noise and SNR Analysis

clc;

clear;

close all;

%% 1. Constants and Input Parameters

q = 1.602e-19;

kB = 1.381e-23;

h = 6.626e-34;

c = 2.998e8;

lambda = 1550e-9;

eta = 0.85;

Id = 5e-9;

T = 300;

RL = 50;

Fn = 2;

B = 1e9;

Pdbm = -60:0.1:0;

P = 10.^((Pdbm-30)/10);

%% 2. Responsivity and Signal Current

R = eta*q*lambda/(h*c);

Ip = R*P;

%% 3. Noise Variances and SNR

var_shot = 2*q*Ip*B;

var_dark = 2*q*Id*B;

var_th = 4*kB*T*B*Fn/RL;

var_tot = var_shot + var_dark + var_th;

SNR_dB = 10*log10(Ip.^2 ./ var_tot);

%% 4. Minimum Detectable Power (SNR = 1)

a = R^2;

b = -2*q*B*R;

cc = -(var_dark + var_th);

Pmin = (-b + sqrt(b^2 - 4*a*cc))/(2*a);

PmindBm = 10*log10(Pmin) + 30;

%% 5. Display Main Results

k = find(abs(Pdbm + 30) < 1e-9, 1);

fprintf('Responsivity R = %.4f A/W\n', R);

fprintf('Thermal variance = %.3e A^2\n', var_th);

fprintf('Dark variance = %.3e A^2\n', var_dark);

fprintf('Shot variance (-30 dBm) = %.3e A^2\n', var_shot(k));

fprintf('SNR at -30 dBm = %.2f dB\n', SNR_dB(k));

fprintf('Pmin (SNR = 1) = %.2f dBm (%.3f nW)\n\n', ...

PmindBm, Pmin*1e9);

%% 6. Test Cases: -50 dBm, -30 dBm and 0 dBm

test_dBm = [-50 -30 0];

for i = 1:length(test_dBm)

Pin_dBm = test_dBm(i);

Pin = 10^((Pin_dBm-30)/10);

I_signal = R*Pin;

shot = 2*q*I_signal*B;

total = shot + var_dark + var_th;

SNR = 10*log10(I_signal^2/total);

fprintf('TEST CASE: POWER (%d dBm)\n', Pin_dBm);

fprintf('Input power = %d dBm\n', Pin_dBm);

fprintf('Signal current = %.3e A\n', I_signal);

fprintf('Shot variance = %.3e A^2\n', shot);

fprintf('Thermal variance = %.3e A^2\n', var_th);

fprintf('Dark variance = %.3e A^2\n', var_dark);

fprintf('Total variance = %.3e A^2\n', total);

fprintf('SNR = %.2f dB\n', SNR);

if SNR < 0

fprintf('Signal is below the 0 dB SNR level.\n\n');

else

fprintf('Signal is at or above the 0 dB SNR level.\n\n');

end

end

%% 7. Graph 1: Noise Variance vs Input Power

figure;

loglog(P, var_th*ones(size(P)), 'LineWidth', 1.5);

hold on;

loglog(P, var_dark*ones(size(P)), 'LineWidth', 1.5);

loglog(P, var_shot, 'LineWidth', 1.5);

loglog(P, var_tot, 'LineWidth', 2);

grid on;

xlabel('Input Power (W)');

ylabel('Noise Variance (A^2)');

title('Noise Variance vs Input Power');

legend('Thermal Noise', 'Dark Current Noise', ...

'Shot Noise', 'Total Noise', ...

'Location', 'best');

%% 8. Graph 2: SNR vs Input Power

figure;

plot(Pdbm, SNR_dB, 'LineWidth', 1.5);

hold on;

yline(0, 'r--', '0 dB SNR');

xline(PmindBm, 'k:', 'Minimum Detectable Power');

grid on;

xlabel('Input Power (dBm)');

ylabel('SNR (dB)');

title('SNR vs Input Power');

legend('SNR', '0 dB SNR', 'Minimum Detectable Power', ...

'Location', 'best');

RESULTS:

RUBRICS :

REFLECTION :

I learned to analyze shot, dark-current, and thermal noise in a photodetector using MATLAB and calculate reposivity, SNR, and minimum detectable optical power. I observed how noiseaffects receiver sensitivity and verified that SNR increases with input optical power.

RESULT :

The study shows how input optical power affects noise and SNR in a PIN photodetector receiver. Performance depends on the power level:

• Low power: thermal noise dominates, giving negative SNR and no reliable detection (below Pmin ≈ −31.16 dBm).

• Moderate power: the signal rises above the thermal noise and the SNR increases at 20 dB/decade.

• High power: SNR becomes large and shot noise grows linearly, approaching the thermal level near +2.9 dBm.

These results help in estimating the receiver sensitivity and in designing low-noise optical receivers.